Sphynx Firmware Version 1.3 Release Notes

This document contains important information about your Sphynx unit.

Updates

Please check our Web site http://www.merging.com periodically for information, patches and updates.

#### Sphynx Firmware Version 1.3

##### Upgrading Your Sphynx Firmware

An updated version of the Sphynx Firmware is now available. This version offers improved performance and bug fixes. It is strongly recommended to upgrade to this new version. This upgrade can be performed easily in the field, by using the firmware removal tool included with the unit. If you are unsure on how to perform this procedure, please contact Merging.

- Disconnect the power supply cable

- Remove the top cover

- Locate the Flash ROM chip (located on the CPU board -is labeled with a sticker “Sphynx V 1.1, or V1.2”.

- Using the removal tool, remove the Flash ROM from the CPU board

- Maintain the tool vertically while removing the Flash.

- Take the new Flash and plug it in the socket (by hand), in place of the old one.

- Take care to ensure that the Firmware chip is inserted in the same orientation as the previous chip.

- Replace the top cover.

##### DISPLAYING FIRMWARE VERSION

When the Sphynx displays the module’s configuration on the Monitor LEDs (at startup or when you press the Monitor switch for 3 sec), it also displays the firmware version on the Level Meters, so you can know your current version without opening the top cover.

It is displayed as follows:

Sphynx V X.Y

X is displayed on the OVL LEDs (RED) from 1 to 8,

Y is displayed on the LVL LEDs (yellow) from 1 to 8.

# Improvements

- Corrects the limitation of V1.2 with respect to the synchronization of AES sources to Sphynx. This version allows again synchronization to an incoming AES-EBU signal connected to an AES-EBU Input module. It doesn't require an AES-EBU Reference signal.

- AES/EBU input modules are now properly muted when the input AES-EBU signal is not properly synchronized as would be the case whenever an AES-EBU signal with a wrong sampling rate or a drift is present at any AES-EBU Input.

- Fixed a potential continuous noise source on AES-EBU Inputs, when in SRC mode, after a Sampling Rate change.

# Current limitations

## Limitations of Firmware Version 1.3:

- Currently, only the S/PDIF coaxial input is operational, but not the S/PDIF output.

- Software Remote Control support is not yet implemented

- The sample rate Mode 'x4' is not available.

## Known problems in Firmware Version 1.3:

- Currently, the Monitoring of input modules does not verify, on power-up, that the last remembered monitor source (selected before the last power-down) is still present. It does not switch to a new default if that particular input module was removed since last power-down.

# User’s Guide

A new version (1.2) of the User’s Guide is available. A PDF version can be downloaded from the following UR: http://www.merging.com/products/sphynx.htm